EXPERIMENT No. 01

Subject - COMPUTER NETWORK

Name - Iqra Shaikh

Batch - 01

Roll No - 24DCO06

AIM - A study of RJ45 & CAT 6 cabling & connection using crimping tool

THEORY -

RJ45

The RJ45 connector is a standard which often connects a computer to a Local Area Network (LAN). It follows the 8P8C (8 Positions, 8 Contacts) configuration, meaning it connects to the eight individual wires .

These connectors usually feature gold-plated contacts to improve electrical conductivity and provide resistance to corrosion, which helps maintain signal integrity over time.

RJ45 connectors are modular and reusable, designed to easily plug int and out of Ethernet ports on routers, switches and other network devices.

There are two wiring standards used for arranging the wires inside the connector: T568A and T568B. These standards ensure compatibility and proper communication across Ethernet networks. While both are functionally similar, T568B is more commonly used in commercial networks.

CAT6

CAT6 cables are high-performance Ethernet cables designed for faster data transfer and higher bandwidth compared to older standards like CAT5 and CAT5e. They support Gigabit Ethernet (up to 1000 Mbps) and can handle 10-Gigabit Ethernet over shorter distances (up to 55 meters).

These cables contain four twisted pairs of copper wires (8 wires total) within a single outer jacket. Each pair is twisted at a different rate to minimize crosstalk and electromagnetic interference (EMI), ensuring reliable signal quality

Crimping Tool:

○A crimping tool is used to attach an RJ45 connector to the end of a CAT6

cable.

○It crimps (presses) the connector onto the wires securely.

○The tool also trims excess wire and ensures proper contact between the

wires and connector pins

### Connection Process

- Strip the CAT6 cable jacket.

- Untwist and arrange wires per T568A or T568B.

- Insert wires into RJ45 connector in the correct order.

- Crimp the connector to secure the wires.

- Plug into an Ethernet port or device.

### Advantages

- High-speed data transfer (up to 10 Gbps).

- Reduced crosstalk and EMI.

- Reliable and durable connections.

- Widely used and cost-effective.

### Disadvantages

- Stiffer than CAT5/CAT5e (less flexible).

- Shorter 10Gbps range (up to 55m).

- Requires precise termination.

### Uses

- Home and office networking

- Data centers

- Video streaming

- VoIP and IP cameras

- Gaming and multimedia setups

Conclusion -

RJ45 connectors and CAT6 cables are key components of modern Ethernet networks. RJ45 provides reliable, modular connections using standardized wiring (T568A/B), while CAT6 cables offer high-speed data transfer with reduced interference. Together, they ensure fast, stable, and efficient network communication.